Based purely on what's wired into the register bank (analytics_top.v), here's what a fully-functioning report from this design would actually look like, and what you could learn from it.

What a report would contain

The 32-register bank splits cleanly into two halves: raw counts (what happened) and derived metrics (what it means). If you read out all 32 registers after a program finishes, it would look like this — illustrative example, not a real simulation run:

Raw activity counters (registers 0–17)

| Reg | Name | Example value | What it tells you |
| --- | --- | --- | --- |
| 0 | cycle_cnt | 5,000 | Total cycles the program took |
| 1 | inst_ret_cnt | 3,200 | Instructions actually completed |
| 2 | branch_cnt | 400 | Branch instructions retired |
| 3 | load_cnt | 600 | Load instructions retired |
| 4 | store_cnt | 250 | Store instructions retired |
| 5 | raw_haz_cnt | 310 | Cycles lost to data dependency hazards |
| 6 | ldu_haz_cnt | 180 | Cycles lost to load-use stalls |
| 7 | ctrl_haz_cnt | 95 | Cycles lost to branch/control hazards |
| 8 | stall_cnt | 585 | Total stall cycles |
| 9 | bubble_cnt | 95 | Pipeline bubbles inserted |
| 10 | flush_cnt | 95 | Pipeline flushes (mis-taken branches) |
| 11–12 | fwd_opp_cnt / fwd_succ_cnt | 310 / 290 | How often forwarding was needed vs. worked |
| 13–17 | if/id/ex/mem/wb_active_cnt | varies | How busy each pipeline stage was |

Derived metrics (registers 18–30)

| Reg | Name | Example value | Meaning |
| --- | --- | --- | --- |
| 18 | metric_ipc | 64 (=0.64) | Instructions per cycle |
| 19 | metric_cpi | 156 (=1.56) | Cycles per instruction |
| 20 | metric_pipe_eff | 64% | How close to ideal 1-instruction-per-cycle throughput |
| 21–25 | metric_util_if/id/ex/mem/wb | 92%, 85%, 78%, 45%, 64% | % of cycles each stage was doing real work |
| 26 | metric_fwd_eff | 93% | % of hazards resolved by forwarding instead of stalling |
| 27–29 | metric_haz_raw/ldu/ctrl_dist | 53%, 31%, 16% | Which hazard type dominates your stalls |
| 30 | metric_mem_ratio | 26% | What fraction of instructions touch memory |

How this helps evaluate the processor

This turns "does it work?" into "how well does it work, and why?" — the kind of data you'd otherwise only get from a hand-built simulation script:

- Overall performance grade: IPC/CPI give you a single number to track as you make design changes — did the last RTL tweak actually speed things up?

- Bottleneck diagnosis: The per-stage utilization numbers tell you which pipeline stage is the traffic jam. If MEM sits at 45% while IF sits at 92%, memory access isn't your bottleneck — something upstream is.

- Root-causing stalls: The hazard distribution tells you which hazard type to spend your engineering effort fixing. If control hazards dominate, better branch handling helps more than more forwarding paths would.

- Judging your own forwarding logic: Forward efficiency tells you whether the bypass network you built is actually catching most hazards, or whether stalls are still slipping through it.

- Workload characterization: Memory ratio and branch count tell you whether a given test program is compute-heavy, memory-heavy, or branch-heavy — useful for picking representative benchmarks.

Questions it's capable of answering

- "Is my processor compute-bound or memory-bound for this workload?"

- "Which pipeline stage is the throughput bottleneck?"

- "Are branch mispredictions or load-use stalls costing me more cycles?"

- "Is my forwarding/bypass logic actually effective, or are hazards still stalling the pipeline?"

- "How far is my design from ideal 1.0 IPC, and is that gap getting better or worse as I change the RTL?"

- "For this specific program, what's the real-world CPI I should quote in a spec sheet?"

The catch, restated

All of this is what the design is capable of based on the RTL as written — it's a legitimate, fairly complete performance-monitoring capability. But as covered earlier, it's currently unverified: no testbench has confirmed a single one of these 32 registers actually reports a correct value, and there's no way yet to read them out of the chip. So today this is a capability that exists on paper, not one you could hand someone a trustworthy report from.