Configurable Processor Preformance Analytics IPThe PMU/event counters become one module inside the IP, while the overall IP provides configurable, mode-selectable processor analytics.

Project Title

Configurable Processor Analytics  IP.

Project Description

Modern processors include Performance Monitoring Units (PMUs) that count hardware events such as CPU cycles, retired instructions, branch events, and cache misses. These counters provide raw numerical data, but processor designers still spend significant time analyzing waveforms, logs, and scripts to determine why performance is limited.

This project proposes a Configurable Processor Analytics Soft IP that connects to an  processor without modifying its functionality. The IP observes pipeline signals, automatically measures pipeline-specific events, computes performance metrics, and generates a structured runtime performance report. The same synthesizable IP can be demonstrated in simulation and on FPGA.

The proposed IP supports two operating modes. Architectural Profiling Mode provides software-oriented statistics such as IPC, CPI, instruction mix, memory accesses, and branch behavior. Microarchitectural Analysis Mode provides pipeline-oriented metrics including hazard classification, forwarding efficiency, pipeline utilization, and stall analysis. This allows the same IP to be useful during processor development as well as FPGA-based application profiling.

The objective is not to replace existing PMUs, but to provide configurable, runtime-selectable analytics for educational, research, and application-development use cases where such analysis is usually performed manually.

Existing Work

| Reference | Measures | Link |
| --- | --- | --- |
| RISC-V Hardware Performance Monitor (Official) | CPU cycles, instructions retired, configurable hardware events | https://docs.riscv.org/reference/isa/privileged.html |
| CORE-V CV32E41P PMU | Cycles, instructions, load/jump hazards, stall cycles, loads, stores, branches, branch taken, IF stall | https://docs.openhwgroup.org/projects/cv32e41p-user-manual/ |
| Linux Perf | Software profiling using PMU counters | https://perf.wiki.kernel.org/ |

Existing On-chip PMUs

Commercial and open processors already include on-chip PMUs that expose a fixed set of hardware events:

- Intel PMU

- ARM PMU

- RISC-V HPM (Hardware Performance Monitor)

These PMUs typically collect:

- Cycles

- Instructions

- Cache misses

- Branch misses

They expose useful architectural-level event counts, but do not classify hazards, measure forwarding efficiency, or report pipeline-stage utilization.The designer must still manually inspect waveforms and interpret signal behavior. This gap motivates a reusable, configurable analytics IP that performs this interpretation automatically.

Limitation of Existing PMUs and Debug Tools

Existing PMUs mainly provide raw event counters. Example:

Cycles          = 125000

Instructions    = 100000

Load Stall      = 2500

Pipeline Stall  = 4000

The designer must still answer:

- Why is IPC low?

- Which pipeline stage is the bottleneck?

- Is forwarding effective?

- Which hazard contributes most?

- Which optimization should be investigated?

Typically this analysis is performed using:

- Verdi

- Questa

- GTKWave

- Python

- Excel

This process consumes significant engineering time. In addition:

- Existing PMUs expose hardware events but provide limited pipeline-level analytics.

- Existing FPGA debug tools require manual signal inspection.

- There is no reusable, configurable analytics IP focused on pipeline behavior that also serves application-level profiling needs.

Proposed Solution

Instead of only counting events, the proposed IP performs event collection, mode-selectable analysis, and runtime statistics generation, then generates a structured report:

Processor

│

wrapper /   Interface

│

Configurable Analytics IP

│

UART / AXI / APB

│

Terminal / Software

Existing Report (Raw Counters)

Cycles                : 125000

Instructions          : 100000

IPC                   : 0.80

Pipeline Stall        : 4200

Load Stall            : 2400

Branches              : 8500

Engineer still performs manual analysis.

Comparison

| Existing PMU | Proposed Configurable Analytics IP |
| --- | --- |
| Counts hardware events | Counts hardware events |
| Reports raw counters | Reports analyzed, mode-selectable metrics |
| Architectural focus only | Architectural AND pipeline-microarchitecture focus |
| Manual post-analysis | Automatic metric generation |
| Software profiling | Processor architecture and application evaluation |
| Generic processor statistics | Configurable, runtime-selectable statistics |

Generic Block Diagram

PROCESSOR

|

|

Pipeline Event Signals

|

v

+----------------------------+

| Configurable Analytics IP  |

|----------------------------|

| Event Collector            |

| Event Counters             |

|----------------------------|

| Analytics Engine           |

|            v               |

| Mode Controller            |

|            v               |

| Register Bank              |

|            v               |

| Interface                  |

+-------------+--------------+

|

------------------------------

|                            |

v                            v

Simulation                  FPGA

($display)               UART / AXI / APB

Mode selection now drives the analytics pipeline: the Analytics Engine's output is routed by the Mode Controller before results are latched into the Register Bank and formatted by the Interface/Report Generator.

Modules

| Module | Function |
| --- | --- |
| Event Collector | Observes pipeline valid, stall, flush, forwarding, hazard, and branch signals |
| Event Counters | Counts cycles, instructions, hazards, branches, forwarding events, pipeline stall cycles |
| Analytics Engine | Computes IPC, pipeline utilization, hazard distribution, forwarding efficiency, stall breakdown, instruction mix |
| Mode Controller | Selects Architectural Profiling Mode or Microarchitectural Analysis Mode through a configuration register |
| Register Bank | Stores computed statistics |
| Report Generator | Formats the selected mode's statistics for UART, AXI/APB, or simulation output |

Interface

Simulation:  Testbench -> $display

FPGA:  UART or AXI / APB

What Is Our Innovation?

Instead of implementing only hardware counters, the proposed IP introduces a configurable analytics architecture capable of generating different performance reports depending on the selected operating mode.

The same synthesized IP can be configured at runtime for:

- Architectural Profiling

- Microarchitectural Analysis

...without modifying the processor RTL. Instead of requiring external scripts and manual waveform inspection, the IP automatically provides standardized, mode-selectable analytics suitable for simulation and FPGA execution.

Who Will Use This IP?

- CPU Architects

- RTL Designers

- Verification Engineers

- Universities

- Research Labs

Benefits

- Faster processor evaluation

- Standardized, mode-selectable performance reporting

- Easier comparison between processor versions

- Reduced manual analysis effort, Reduces time.

- Reusable synthesizable IP across multiple designs and use cases

Configuration Register

A single configuration bit selects the IP's operating mode at runtime:

| Bit 0 Value | Selected Mode |
| --- | --- |
| 0 | Architectural Profiling Mode |
| 1 | Microarchitectural Analysis Mode |

Software writes

|

Register changes

|

Different report generated

Because the operating mode is selected through this register rather than fixed in RTL, the same synthesized IP instance can serve both processor-development and application-profiling workflows without resynthesis.

Proposed Performance Analysis Report

Instead of a single fixed report, the IP generates one of two mode-specific reports, selected by the Configuration Register.

Mode 0 — Architectural Profiling

Software-oriented statistics, useful for application-level performance evaluation:

- Cycles

- Instructions

- IPC

- CPI

- Instruction Mix

- Branch Density

- Memory Access

- CPU Utilization

Mode 1 — Microarchitectural Analysis

Pipeline-oriented statistics, useful for processor design and verification:

- Hazards

- Forwarding

- Pipeline Utilization

- Bubble Count

- Flush Count

- Average Stall

- Pipeline Efficiency

Example Report Output

====================================

PIPELINE PERFORMANCE REPORT

Mode: Microarchitectural Analysis

====================================

Cycles                 :125000

Instructions           :100000

IPC                    :0.80

------------------------------------

Hazard Analysis

Load Hazards           :1950

RAW Hazards            :180

Control Hazards        :270

------------------------------------

Forwarding Analysis

Forward Opportunities  :1950

Forward Success        :1880

Forward Failure        :70

Forward Efficiency     :96%

------------------------------------

Pipeline Utilization

IF                     :98%

ID                     :97%

EX                     :96%

MEM                    :62%

WB                     :95%

------------------------------------

Instruction Mix

Arithmetic             :45%

Memory                 :36%

Branch                 :15%

Others                 :4%

====================================

Detailed Metric Reference

1. Execution Statistics (Mode 0)

| Parameter | Existing | Description |
| --- | --- | --- |
| Total Cycles | Yes | Total execution cycles |
| Instructions Retired | Yes | Successfully completed instructions |
| CPI | Calculated | Cycles / Instructions |
| IPC | Calculated | Instructions / Cycle |
| Execution Time | New | Cycles x Clock Period |

2. Instruction Statistics (Mode 0)

| Parameter | Existing | Description |
| --- | --- | --- |
| Arithmetic Instructions | New | ADD, SUB, XOR, OR... |
| Logical Instructions | New | AND, OR, XOR |
| Shift Instructions | New | SLL, SRL, SRA |
| Load Instructions | Yes | Number of loads |
| Store Instructions | Yes | Number of stores |
| Branch Instructions | Yes | Number of branches |
| Jump Instructions | Yes | Number of jumps |
| CSR Instructions | New | CSR accesses |
| NOP Count | New | Bubble/NOP instructions |
| Instruction Mix (%) | New | Percentage of each instruction class |

3. Pipeline Utilization (Mode 1)

| Parameter | Existing | Description |
| --- | --- | --- |
| IF Utilization | New | IF active cycles / Total cycles |
| ID Utilization | New | ID active cycles |
| EX Utilization | New | EX active cycles |
| MEM Utilization | New | MEM active cycles |
| WB Utilization | New | WB active cycles |

4. Hazard Analysis (Mode 1)

| Parameter | Existing | Description |
| --- | --- | --- |
| Load-use Hazard | Yes | Existing counter |
| RAW Hazard | New | Read-after-write dependencies |
| Control Hazard | New | Branch/Jump hazards |
| Structural Hazard | New | If implemented |
| Hazard Distribution | New | Percentage of each hazard |

5. Stall Analysis (Mode 1)

| Parameter | Existing | Description |
| --- | --- | --- |
| Total Stall Cycles | Yes | Existing PIPE_STALL |
| Load Stall Cycles | Yes | Existing LD_STALL |
| Branch Stall Cycles | New | Branch-related stalls |
| Average Stall Length | New | Stall cycles / Stall events |
| Stall Percentage | New | Stall cycles / Total cycles |

6. Forwarding Analysis (Mode 1)

| Parameter | Existing | Description |
| --- | --- | --- |
| Forward Opportunities | New | Total dependencies detected |
| Forward Used | New | Forwarding activated |
| Forward Success | New | Hazard solved by forwarding |
| Forward Failure | New | Stall still required |
| Forwarding Efficiency | New | Success / Opportunities |

7. Branch Analysis (Mode 0 )

| Parameter | Existing | Description |
| --- | --- | --- |
| Branch Count | Yes | Existing |
| Branch Taken | Yes | Existing |
| Branch Not Taken | New | Calculated |
| Branch Taken Ratio | New | Percentage |
| Flush Count | New | Pipeline flushes |

8. Memory Analysis (Mode 0)

| Parameter | Existing | Description |
| --- | --- | --- |
| Load Count | Yes | Existing |
| Store Count | Yes | Existing |
| Memory Instruction Ratio | New | Load+Store / Instructions |
| Memory Busy Cycles | New | MEM stage active cycles |

9. Pipeline Efficiency (Mode 1)

| Parameter | Existing | Description |
| --- | --- | --- |
| Useful Cycles | New | Cycles executing useful work |
| Bubble Cycles | New | Empty pipeline cycles |
| Flush Cycles | New | Pipeline flushes |
| Pipeline Efficiency | New | Useful / Total cycles |

Report 1 – Existing PMU Report (Current Systems)

====================================================

STANDARD PROCESSOR PERFORMANCE REPORT

====================================================

Execution Statistics

--------------------

Clock Cycles                 : 125,000

Instructions Retired         : 100,000

IPC                          : 0.80

Hardware Event Counters

-----------------------

Load Instructions            : 24,500

Store Instructions           : 11,200

Branch Instructions          : 8,500

Branch Taken                 : 6,300

Pipeline Events

---------------

Pipeline Stall Cycles        : 4,200

Load Stall Cycles            : 2,900

Jump Stall Cycles            : 480

Instruction Fetch Stall      : 320

====================================================

Problems with Existing Report

The engineer still has to answer:

- Which pipeline stage is the bottleneck?

- Are stalls mainly due to hazards or memory?

- Is forwarding working efficiently?

- How balanced is the pipeline?

- Which optimization should be attempted first?

To answer these questions, engineers inspect waveforms or analyze logs using external tools.

Report 2 – Proposed Configurable Processor Analytics IP

=========================================================

CONFIGURABLE PROCESSOR ANALYTICS REPORT

=========================================================

Execution Statistics

--------------------

Clock Cycles                 : 125,000

Instructions Retired         : 100,000

IPC                          : 0.80

CPI                          : 1.25

=========================================================

Instruction Profile

=========================================================

Arithmetic Instructions      : 41%

Memory Instructions          : 36%

Branch Instructions          : 15%

Logical Instructions         : 6%

Other Instructions           : 2%

=========================================================

Pipeline Utilization

=========================================================

Instruction Fetch (IF)       : 98%

Instruction Decode (ID)      : 96%

Execute (EX)                 : 94%

Memory (MEM)                 : 61%

Write Back (WB)              : 97%

=========================================================

Hazard Analysis

=========================================================

RAW Hazards                  : 1,250

Load-use Hazards             : 820

Control Hazards              : 190

Structural Hazards           : 0

Hazard Distribution

RAW                          : 55%

Load-use                     : 36%

Control                      : 9%

=========================================================

Forwarding Analysis

=========================================================

Forward Opportunities        : 1,250

Forward Success              : 1,180

Forward Failure              : 70

Forwarding Efficiency        : 94.4%

=========================================================

Pipeline Stall Analysis

=========================================================

Total Stall Cycles           : 4,200

Average Stall Length         : 1.3 cycles

Bubble Count                 : 160

Pipeline Flush Count         : 42

=========================================================

Architectural Statistics

=========================================================

Memory Access Ratio          : 36%

Branch Density               : 15%

CPU Utilization              : 91%

=========================================================

Report Mode

=========================================================

Microarchitectural Analysis Mode

| Category | Metric | Description | Insight Obtained | Optimization Decision |
| --- | --- | --- | --- | --- |
| Execution | Total Cycles | Total execution cycles of the program | Overall execution cost | Compare different processor versions |
|  | Instructions Retired | Successfully completed instructions | Work completed by the processor | Used with cycles to evaluate efficiency |
|  | IPC | Instructions executed per cycle | Low IPC indicates performance bottlenecks | Investigate stalls, hazards, or memory delays |
|  | CPI | Cycles required per instruction | High CPI indicates inefficient execution | Optimize pipeline and reduce stall cycles |
| Pipeline Utilization | IF Utilization | Percentage of active Instruction Fetch cycles | Detect idle instruction fetch stage | Improve instruction fetch or branch handling |
|  | ID Utilization | Percentage of active Decode cycles | Identify decode-stage bottlenecks | Optimize decode/control logic |
|  | EX Utilization | Percentage of active Execute cycles | Measure ALU usage | Improve execution pipeline balance |
|  | MEM Utilization | Percentage of active Memory cycles | Detect memory bottleneck | Optimize memory subsystem or reduce memory accesses |
|  | WB Utilization | Percentage of active Write-Back cycles | Identify write-back congestion | Improve register write scheduling |
| Hazard Analysis | RAW Hazard | Read-after-write dependencies detected | Frequency of data dependencies | Improve forwarding or instruction scheduling |
|  | Load-use Hazard | Hazards caused by load dependencies | Pipeline waiting for memory data | Add forwarding or reduce dependent loads |
|  | Control Hazard | Hazards caused by branches/jumps | Branch penalty and pipeline disruption | Improve branch prediction or branch handling |
| Stall Analysis | Total Stall Cycles | Total cycles lost due to pipeline stalls | Overall performance loss | Identify severity of pipeline inefficiency |
|  | Load Stall Cycles | Stall cycles caused by memory loads | Memory-related bottleneck | Optimize memory latency or forwarding |
|  | Branch Stall Cycles | Stall cycles caused by branches | Branch handling inefficiency | Improve branch prediction strategy |
|  | Bubble Count | Number of empty pipeline cycles | Pipeline under-utilization | Reduce unnecessary stalls and hazards |
|  | Flush Count | Number of pipeline flushes | Cost of branch mispredictions or redirects | Optimize control-flow logic |
| Forwarding Analysis | Forward Opportunities | Total dependency situations where forwarding could help | Potential forwarding usage | Evaluate forwarding requirement |
|  | Forward Success | Hazards successfully resolved by forwarding | Effectiveness of forwarding hardware | Verify forwarding implementation |
|  | Forward Failure | Hazards not solved by forwarding | Remaining performance loss | Improve forwarding paths or hazard logic |
|  | Forward Efficiency | Percentage of successful forwarding | Overall forwarding performance | Quantify benefit of forwarding optimization |
| Memory Analysis | Load Count | Number of load instructions | Memory read intensity | Optimize memory access patterns |
|  | Store Count | Number of store instructions | Memory write intensity | Evaluate memory workload |
|  | Memory Instruction Ratio | Percentage of memory instructions | Indicates memory-bound applications | Decide whether memory subsystem needs optimization |
| Overall Performance | Pipeline Efficiency | Useful execution cycles / Total cycles | Overall pipeline utilization | Compare processor versions and measure optimization effectiveness |

Example of how designers use these metrics

Case 1: Memory Bottleneck

MEM Utilization      = 95%

Load Stall Cycles    = 1800

Memory Ratio         = 42%

Pipeline Efficiency  = 68%

Insight:

- Memory stage is overloaded.

- Many cycles are lost waiting for memory.

Optimization:

- Add cache (future work).

- Improve memory interface.

- Optimize software to reduce load/store operations.

Case 2: Forwarding Problem

RAW Hazard          = 950

Forward Success     = 150

Forward Failure     = 800

Forward Efficiency  = 15%

Insight:

- Most data hazards are not being resolved by forwarding.

Optimization:

- Improve forwarding network.

- Modify hazard detection logic.

Case 3: Branch Bottleneck

Control Hazard      = 420

Branch Stall        = 600

Flush Count         = 300

IF Utilization      = 62%

Insight:

- Frequent branch penalties are preventing the fetch stage from staying busy.

Optimization:

- Improve branch prediction.

- Reduce branch penalties.

Case 4: Well-Optimized Processor

IPC                 = 0.96

Pipeline Efficiency = 94%

Forward Efficiency  = 98%

MEM Utilization     = 83%

Bubble Count        = 15

Insight:

- Pipeline is balanced.

- Forwarding is effective.

- Few wasted cycles.

Optimization:

- No major bottleneck; only fine-tuning is needed.

Why this is innovative

Existing PMUs typically report raw counts such as:

Cycles = 125000

Instructions = 100000

Load Stalls = 4200

The engineer still has to inspect waveforms and logs to determine the bottleneck.

Your Analytics IP reports interpretable metrics such as:

- Pipeline utilization

- Hazard distribution

- Forwarding efficiency

- Pipeline efficiency

These metrics directly answer questions like:

- Which pipeline stage is limiting performance?

- Are stalls mainly due to memory, data hazards, or branches?

- Is the forwarding unit working effectively?

- How much performance is being lost due to pipeline inefficiencies?

- Which subsystem should be optimized first?

https://ieeexplore.ieee.org/document/9821043